### 1. The Network Scheme (N1 - N8)

Yes, the network addresses MUST be different.

Even though you are using the same VLAN IDs (10, 20, 30) inside every campus, the IP Subnets must be unique. If Campus 1 Admin has 192.168.10.0 and Campus 2 Admin also has 192.168.10.0, the routers will get confused and traffic will drop.

#### A. Internal Campus Networks (The LANs)

These are the networks inside your Campuses (N1, N2, N3, N4).

| Label | Campus | VLAN 10 (Admin) | VLAN 20 (Student) | VLAN 30 (Lab) |
| --- | --- | --- | --- | --- |
| N1 | C1 (Top Left) | 192.168.10.0 /24 | 192.168.20.0 /24 | 192.168.30.0 /24 |
| N2 | C2 (Bot Left) | 192.168.40.0 /24 | 192.168.50.0 /24 | 192.168.60.0 /24 |
| N4 | C3 (Bot Right) | 192.168.70.0 /24 | 192.168.80.0 /24 | 192.168.90.0 /24 |
| N3 | C4 (Top Right) | 192.168.100.0/24 | 192.168.110.0/24 | 192.168.120.0/24 |

#### B. Inter-Campus Links (The WAN)

These are the fiber connections between the L3 Switches (N5, N6, N7, N8).

We use /30 because we only need 2 IPs per cable.

| Label | Connection | Network ID | IP 1 (First Switch) | IP 2 (Second Switch) |
| --- | --- | --- | --- | --- |
| N5 | C1 <-> C4 | 10.0.0.0 /30 | .1 (C1) | .2 (C4) |
| N6 | C1 <-> C2 | 10.0.0.4 /30 | .6 (C1) | .5 (C2) |
| N7 | C2 <-> C3 | 10.0.0.12 /30 | .14 (C2) | .13 (C3) |
| N8 | C3 <-> C4 | 10.0.0.8 /30 | .10 (C3) | .9 (C4) |