# CCP REPORT

# Secure Multi-Campus MAN using OSPF & Inter-VLAN Routing

COURSE: Routing & Switching Theory

SUBMITTED TO: Ali Ahmed Siddiqui

GROUP MEMBERS:

| NAMES | REG ID |
| --- | --- |
| FIZZA BAIG | 63503 |
| ABDUL SARIM KHAN | 62527 |
| MUHAMMAD JAHANZAIB | 63843 |
| KOMAL JALEES | 63614 |

# Summary

This project demonstrates the design and implementation of a scalable Metropolitan Area Network (MAN) connecting four distinct campuses of Iqra University (Main, North, Airport, and Gulshan). The network is designed with a strict Zero Trust security model for students. While administrative staff enjoys seamless inter-campus connectivity, student and lab access is rigorously restricted. The solution utilizes Layer 3 Multilayer Switches for routing, OSPF for WAN connectivity, and Access Control Lists (ACLs) to isolate student traffic from all other network segments, including the Data Center and other campuses.

# Project Motivation & Objectives

Large educational institutions require robust networks that can handle distinct user groups with different access privileges.

- Scalability: Implementation of OSPF allows the network to adapt automatically to topology changes across the four campuses.

- Strict Segmentation: The primary objective is to prevent student users from accessing unauthorized resources, specifically the Admin network and central servers.

- Bandwidth Conservation: By blocking student traffic from traversing the WAN links, we ensure that inter-campus bandwidth is reserved exclusively for administrative use.

# Network Topology & Design

## Overview

The topology consists of four campuses connected in a mesh/ring architecture using high-speed Gigabit links. Each campus features a Core Multilayer Switch that acts as the gateway and policy enforcer.

## Campus Identifiers

- Main Campus

- North Nazimabad Campus

- Gulshan Campus

- Airport Campus

## IP Addressing Scheme (VLSM)

We employed a structured IP scheme using /24 subnets for LANs and /30 subnets for WAN links.

| Campus | VLAN 1 (Admin) | VLAN 20 (Student) | VLAN 30 (Lab) |
| --- | --- | --- | --- |
| Main | 192.168.10.0/24 | 192.168.20.0/24 | 192.168.30.0/24 |
| North | 192.168.40.0/24 | 192.168.50.0/24 | 192.168.60.0/24 |
| Gulshan | 192.168.70.0/24 | 192.168.80.0/24 | 192.168.90.0/24 |
| Airport | 192.168.100.0/24 | 192.168.110.0/24 | 192.168.120.0/24 |

# Configuration & Implementation

## VLAN Configuration & Inter-VLAN Routing

We utilized Switched Virtual Interfaces (SVIs) on Multilayer Switches instead of legacy Router-on-a-Stick.

- VLAN 1 (Admin): The only VLAN permitted to route traffic between campuses.

- VLAN 20 (Student) & VLAN 30 (Lab): These VLANs are configured for local access only.

## Dynamic Routing (OSPF)

OSPF (Open Shortest Path First) was configured on all Core Switches in Area 0. This provides full routing tables to the switches, allowing them to make intelligent forwarding decisions. However, security policy restrictions (ACLs) take precedence over these routes for student traffic.

## DHCP Services

To optimize network performance and provide fault tolerance, local DHCP services are distributed across each campus. Each campus utilizes its local Admin Server as the DHCP authority for its specific branch.

- DHCP Relay Implementation: The command ip helper-address is configured on all local Student and Lab VLAN interfaces (VLAN 20 and 30) on the Core Multilayer Switch.

- Logical Flow: This configuration allows the Core Switch to intercept local DHCP broadcast requests from students and labs and forward them as unicast packets to the local Admin Server.

- Redundancy & Performance: By localizing DHCP, we ensure that if a WAN link fails, local students can still obtain IP addresses from their campus server.

- Server Assignment:

  - Main Campus Server: 192.168.10.10

  - North Campus Server: 192.168.40.10

  - Gulshan Campus Server: 192.168.70.10

  - Airport Campus Server: 192.168.100.10

## Security & Access Control Lists (ACLs)

A strict Extended ACL named CAMPUS_ISOLATION is applied inbound on Student and Lab interfaces.

Security Policies Implemented:

- Intra-VLAN Communication: ALLOWED. (e.g., Student PC to Student PC in the same room) via Layer 2 switching.

- DHCP Acquisition: ALLOWED. UDP ports 67/68 are specifically permitted so devices can get an IP.

- Admin/Server Access: BLOCKED. Access to the Admin IP range (VLAN 1) is denied.

- Inter-Campus Access: BLOCKED. Access to other campuses via WAN is denied.

## ACL Logic:

The ACL explicitly permits DHCP traffic (permit udp any any eq bootps) and then utilizes a deny statement to block all other inter-network traffic.

# Topology Snapshot

Figure 1- Topology

# Running Configurations

## Main Campus Multilayer Switch

version 12.2(37)SE1 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Core_Main ! ip routing ! spanning-tree mode pvst ! interface FastEthernet0/1 switchport trunk encapsulation dot1q switchport mode trunk ! interface FastEthernet0/2 switchport trunk encapsulation dot1q switchport mode trunk ! interface FastEthernet0/3 switchport trunk encapsulation dot1q switchport mode trunk ! interface FastEthernet0/4 ! interface FastEthernet0/5 ! interface FastEthernet0/6 ! interface FastEthernet0/7 ! interface FastEthernet0/8 ! interface FastEthernet0/9 ! interface FastEthernet0/10 ! interface FastEthernet0/11 ! interface FastEthernet0/12 ! interface FastEthernet0/13 ! interface FastEthernet0/14 ! interface FastEthernet0/15 ! interface FastEthernet0/16 ! interface FastEthernet0/17 ! interface FastEthernet0/18 ! interface FastEthernet0/19 ! interface FastEthernet0/20 ! interface FastEthernet0/21 ! interface FastEthernet0/22 ! interface FastEthernet0/23 ! interface FastEthernet0/24 ! interface GigabitEthernet0/1 no switchport ip address 10.0.0.1 255.255.255.252 duplex auto speed auto ! interface GigabitEthernet0/2 no switchport ip address 10.0.0.6 255.255.255.252 duplex auto speed auto ! interface Vlan1 ip address 192.168.10.1 255.255.255.0 ! interface Vlan20 mac-address 0060.70e9.1e02 ip address 192.168.20.1 255.255.255.0 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! interface Vlan30 mac-address 0060.70e9.1e03 ip address 192.168.30.1 255.255.255.0 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! router ospf 1 log-adjacency-changes network 192.168.10.0 0.0.0.255 area 0 network 192.168.20.0 0.0.0.255 area 0 network 192.168.30.0 0.0.0.255 area 0 network 10.0.0.0 0.0.0.3 area 0 network 10.0.0.4 0.0.0.3 area 0 ! ip classless ! ip flow-export version 9 ! ! ip access-list extended CAMPUS_ISOLATION permit udp any any eq bootps permit udp any any eq bootpc permit icmp any any echo-reply permit ip 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255 permit ip 192.168.30.0 0.0.0.255 192.168.20.0 0.0.0.255 deny ip any any ! line con 0 ! line aux 0 ! line vty 0 4 login ! end

## North Campus Multilayer Switch

version 12.2(37)SE1 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Core_North ! ip routing ! spanning-tree mode pvst ! interface FastEthernet0/1 ! interface FastEthernet0/2 ! interface FastEthernet0/3 ! interface FastEthernet0/4 ! interface FastEthernet0/5 ! interface FastEthernet0/6 ! interface FastEthernet0/7 ! interface FastEthernet0/8 ! interface FastEthernet0/9 ! interface FastEthernet0/10 ! interface FastEthernet0/11 ! interface FastEthernet0/12 ! interface FastEthernet0/13 ! interface FastEthernet0/14 ! interface FastEthernet0/15 ! interface FastEthernet0/16 ! interface FastEthernet0/17 ! interface FastEthernet0/18 ! interface FastEthernet0/19 ! interface FastEthernet0/20 ! interface FastEthernet0/21 ! interface FastEthernet0/22 ! interface FastEthernet0/23 ! interface FastEthernet0/24 ! interface GigabitEthernet0/1 no switchport ip address 10.0.0.5 255.255.255.252 duplex auto speed auto ! interface GigabitEthernet0/2 no switchport ip address 10.0.0.14 255.255.255.252 duplex auto speed auto ! interface Vlan1 ip address 192.168.40.1 255.255.255.0 ! interface Vlan20 mac-address 0010.1195.7401 ip address 192.168.50.1 255.255.255.0 ip helper-address 192.168.40.10 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! interface Vlan30 mac-address 0010.1195.7402 ip address 192.168.60.1 255.255.255.0 ip helper-address 192.168.40.10 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! router ospf 1 log-adjacency-changes network 192.168.40.0 0.0.0.255 area 0 network 192.168.50.0 0.0.0.255 area 0 network 192.168.60.0 0.0.0.255 area 0 network 10.0.0.4 0.0.0.3 area 0 network 10.0.0.12 0.0.0.3 area 0 ! ip classless ! ip flow-export version 9 ! ip access-list extended CAMPUS_ISOLATION permit udp any any eq bootps permit udp any any eq bootpc permit icmp any any echo-reply permit ip 192.168.50.0 0.0.0.255 192.168.60.0 0.0.0.255 permit ip 192.168.60.0 0.0.0.255 192.168.50.0 0.0.0.255 deny ip any any ! line con 0 ! line aux 0 ! line vty 0 4 login ! end

## Gulshan Campus Multilayer Switch

version 12.2(37)SE1 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Core_Gulshan ! ip routing ! spanning-tree mode pvst ! interface FastEthernet0/1 ! interface FastEthernet0/2 ! interface FastEthernet0/3 ! interface FastEthernet0/4 ! interface FastEthernet0/5 ! interface FastEthernet0/6 ! interface FastEthernet0/7 ! interface FastEthernet0/8 ! interface FastEthernet0/9 ! interface FastEthernet0/10 ! interface FastEthernet0/11 ! interface FastEthernet0/12 ! interface FastEthernet0/13 ! interface FastEthernet0/14 ! interface FastEthernet0/15 ! interface FastEthernet0/16 ! interface FastEthernet0/17 ! interface FastEthernet0/18 ! interface FastEthernet0/19 ! interface FastEthernet0/20 ! interface FastEthernet0/21 ! interface FastEthernet0/22 ! interface FastEthernet0/23 ! interface FastEthernet0/24 ! interface GigabitEthernet0/1 no switchport ip address 10.0.0.13 255.255.255.252 duplex auto speed auto ! interface GigabitEthernet0/2 no switchport ip address 10.0.0.10 255.255.255.252 duplex auto speed auto ! interface Vlan1 ip address 192.168.70.1 255.255.255.0 ! interface Vlan20 mac-address 0060.2f3a.e601 ip address 192.168.80.1 255.255.255.0 ip helper-address 192.168.70.10 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! interface Vlan30 mac-address 0060.2f3a.e602 ip address 192.168.90.1 255.255.255.0 ip helper-address 192.168.70.10 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! router ospf 1 log-adjacency-changes network 192.168.70.0 0.0.0.255 area 0 network 192.168.80.0 0.0.0.255 area 0 network 192.168.90.0 0.0.0.255 area 0 network 10.0.0.12 0.0.0.3 area 0 network 10.0.0.8 0.0.0.3 area 0 ! ip classless ! ip flow-export version 9 ! ip access-list extended CAMPUS_ISOLATION permit udp any any eq bootps permit udp any any eq bootpc permit icmp any any echo-reply permit ip 192.168.80.0 0.0.0.255 192.168.90.0 0.0.0.255 permit ip 192.168.90.0 0.0.0.255 192.168.80.0 0.0.0.255 deny ip any any ! line con 0 ! line aux 0 ! line vty 0 4 login ! end

## Airport Campus Multilayer Switch

version 12.2(37)SE1 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Core_Airport ! ip routing ! spanning-tree mode pvst ! interface FastEthernet0/1 ! interface FastEthernet0/2 ! interface FastEthernet0/3 ! interface FastEthernet0/4 ! interface FastEthernet0/5 ! interface FastEthernet0/6 ! interface FastEthernet0/7 ! interface FastEthernet0/8 ! interface FastEthernet0/9 ! interface FastEthernet0/10 ! interface FastEthernet0/11 ! interface FastEthernet0/12 ! interface FastEthernet0/13 ! interface FastEthernet0/14 ! interface FastEthernet0/15 ! interface FastEthernet0/16 ! interface FastEthernet0/17 ! interface FastEthernet0/18 ! interface FastEthernet0/19 ! interface FastEthernet0/20 ! interface FastEthernet0/21 ! interface FastEthernet0/22 ! interface FastEthernet0/23 ! interface FastEthernet0/24 ! interface GigabitEthernet0/1 no switchport ip address 10.0.0.2 255.255.255.252 duplex auto speed auto ! interface GigabitEthernet0/2 no switchport ip address 10.0.0.9 255.255.255.252 duplex auto speed auto ! interface Vlan1 ip address 192.168.100.1 255.255.255.0 ! interface Vlan20 mac-address 00d0.ba9e.3401 ip address 192.168.110.1 255.255.255.0 ip helper-address 192.168.100.10 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! interface Vlan30 mac-address 00d0.ba9e.3402 ip address 192.168.120.1 255.255.255.0 ip helper-address 192.168.100.10 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! router ospf 1 log-adjacency-changes network 192.168.100.0 0.0.0.255 area 0 network 192.168.110.0 0.0.0.255 area 0 network 192.168.120.0 0.0.0.255 area 0 network 10.0.0.0 0.0.0.3 area 0 network 10.0.0.8 0.0.0.3 area 0 ! ip classless ! ip flow-export version 9 ! ip access-list extended CAMPUS_ISOLATION permit udp any any eq bootps permit udp any any eq bootpc permit icmp any any echo-reply permit ip 192.168.110.0 0.0.0.255 192.168.120.0 0.0.0.255 permit ip 192.168.120.0 0.0.0.255 192.168.110.0 0.0.0.255 deny ip any any ! line con 0 ! line aux 0 ! line vty 0 4 login ! end

## Layer 2 Switches

version 12.2(37)SE1 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Core_Airport ! ip routing ! spanning-tree mode pvst ! interface FastEthernet0/1 ! interface FastEthernet0/2 ! interface FastEthernet0/3 ! interface FastEthernet0/4 ! interface FastEthernet0/5 ! interface FastEthernet0/6 ! interface FastEthernet0/7 ! interface FastEthernet0/8 ! interface FastEthernet0/9 ! interface FastEthernet0/10 ! interface FastEthernet0/11 ! interface FastEthernet0/12 ! interface FastEthernet0/13 ! interface FastEthernet0/14 ! interface FastEthernet0/15 ! interface FastEthernet0/16 ! interface FastEthernet0/17 ! interface FastEthernet0/18 ! interface FastEthernet0/19 ! interface FastEthernet0/20 ! interface FastEthernet0/21 ! interface FastEthernet0/22 ! interface FastEthernet0/23 ! interface FastEthernet0/24 ! interface GigabitEthernet0/1 no switchport ip address 10.0.0.2 255.255.255.252 duplex auto speed auto ! interface GigabitEthernet0/2 no switchport ip address 10.0.0.9 255.255.255.252 duplex auto speed auto ! interface Vlan1 ip address 192.168.100.1 255.255.255.0 ! interface Vlan20 mac-address 00d0.ba9e.3401 ip address 192.168.110.1 255.255.255.0 ip helper-address 192.168.100.10 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! interface Vlan30 mac-address 00d0.ba9e.3402 ip address 192.168.120.1 255.255.255.0 ip helper-address 192.168.100.10 ip helper-address 192.168.10.10 ip access-group CAMPUS_ISOLATION in ! router ospf 1 log-adjacency-changes network 192.168.100.0 0.0.0.255 area 0 network 192.168.110.0 0.0.0.255 area 0 network 192.168.120.0 0.0.0.255 area 0 network 10.0.0.0 0.0.0.3 area 0 network 10.0.0.8 0.0.0.3 area 0 ! ip classless ! ip flow-export version 9 ! ip access-list extended CAMPUS_ISOLATION permit udp any any eq bootps permit udp any any eq bootpc permit icmp any any echo-reply permit ip 192.168.110.0 0.0.0.255 192.168.120.0 0.0.0.255 permit ip 192.168.120.0 0.0.0.255 192.168.110.0 0.0.0.255 deny ip any any ! line con 0 ! line aux 0 ! line vty 0 4 login ! end

# Testing & Verification Results

The following connectivity tests confirm that the security policies are active and functioning correctly. A key distinction was observed between DHCP functionality (which works) and Ping functionality (which is blocked for security).

## Connectivity Table

| # | Source Device | Destination Device | Type | Status |
| --- | --- | --- | --- | --- |
| 1 | Admin PC | Student PC | Local/WAN | SUCCESS |
| 2 | Admin PC | Lab PC | Local/WAN | SUCCESS |
| 3 | Student PC | Admin PC | Local/WAN | FAILED |
| 4 | Student PC | Lab PC | Local | SUCCESS |
| 5 | Student PC | Lab PC | Inter-Campus | FAILED |
| 6 | Student PC | Student PC | Inter-Campus | FAILED |
| 7 | Student PC | Student PC | Local | SUCCESS |
| 8 | Lab PC | Student PC | Local | SUCCESS |
| 9 | Lab PC | Student PC | Inter-Campus | FAILED |
| 10 | Lab PC | Admin PC | Local/WAN | FAILED |
| 11 | Lab PC | Lab PC | Local | SUCCESS |
| 12 | Lab PC | Lab PC | Inter-Campus | FAILED |

## Security Exception: DHCP Protocol Permit

During the implementation phase, a critical challenge was observed where the strict deny ip any any rule blocked DHCP requests. To resolve this while maintaining security, the following specific entries were prioritized in the CAMPUS_ISOLATION ACL:

- 1 permit udp any any eq bootps

- 2 permit udp any any eq bootpc

This allows PCs to obtain an initial IP address even before their identity is established on the network, enabling the Zero Trust model to function without breaking basic network services.

# Conclusion

This project successfully delivered a secure, multi-campus network for Iqra University. By combining OSPF routing with strict Access Control Lists (ACLs), we ensured seamless connectivity for administration while restricting student traffic to local subnets. This design effectively secures critical resources and optimizes bandwidth, resulting in a robust and scalable network infrastructure.